Panasonic Industry Europe introduces world’s smallest 2pole safety relay with power contacts on NO and NC side

Panasonic Industry Europe GmbH

Robert-Koch-Strasse 100

85521 Ottobrunn, Germany

http://industry.panasonic.eu

Press contact:

Moritz Cehak

Email: moritz.cehak@eu.panasonic.com

Phone: +49 89 45354 1228

http://industry.panasonic.eu

SFM relay series offers forcibly guided contacts according to EN61810-3

Munich, January 2020: More than 30 years ago, Panasonic changed the way how to design safety modules by bringing a low profile 8pole safety relay on the market. Now the launch of the new 1a1b SFM relay will start the next phase of the market revolution. With a height of only 7.8mm and reflow soldering capability, SFM provides great design possibilities and is ready for future production processes.

As all Panasonic Industry safety relays, SFM offers power contacts on both NO (6A) and NC (4A) side to control loads like valves directly without another relay in between. Contrarywise, the silver alloy contacts can handle low level loads down to 10V 1mA as well to support e.g. signaling boards in the railway sector. A very high shock and vibration resistance 
≥ 20g, low coil holding power of 100mW, an ambient temperature 
range from -40°C to 85°C and reinforced insulation 
≥ 5.5mm (V=230V overvoltage category III, 6KV) on NO side contribute to an almost unlimited field of applications.

Further information can be found here.

###

About Panasonic Industry Europe

Panasonic Industry Europe GmbH is part of the global Panasonic Group and provides automotive and industrial products and services in Europe. As a partner for the industrial sector, Panasonic researches, develops, manufactures and supplies technologies that support the slogan “A Better Life, A Better World”.

The company’s portfolio covers key electronic components, devices and modules up to complete solutions and production equipment for manufacturing lines across a broad range of industries. Panasonic Industry Europe is part of the global company Panasonic Industrial Solutions. More about Panasonic Industry Europe: http://industry.panasonic.eu

About Panasonic

Panasonic Corporation is a worldwide leader in the development of diverse electronics technologies and solutions for customers in the consumer electronics, housing, automotive, and B2B businesses. The company, which celebrated its 100th anniversary in 2018, has expanded globally and now operates 582 subsidiaries and 87 associated companies worldwide, recording consolidated net sales of US$72.10 billion for the year ended March 31, 2019. Committed to pursuing new value through innovation across divisional lines, the company uses its technologies to create a better life and a better world for its customers. To learn more about Panasonic: http://www.panasonic.com/global.